# 5.1.2 Emergency exit (Hätähyppy)

Instructor is the chief of students ← Listen to his/her instructions!

EMERGENCY EXIT ALTITUDES LEVELS 1–7 (AFF)

- Main canopy, when at normal deployment altitude or above (1600–1400 m).

- Reserve canopy, when below normal deployment altitude (straight to reserve deployment handle without cutaway handle).

EMERGENCY EXIT ALTITUDES STARTING FROM LEVEL 8 (AFF)

- Main canopy above 1 000 meters.

- Reserve canopy below 1 000 meters straight by using reserve handle (no cutaway).

Emergency exit with reserve canopy

Exit always alone by pulling the reserve handle right away after the exit.

Commands and actions when using reserve canopy

- Instructor commands: HÄTÄHYPPY (EMERGENCY EXIT)!  Repeat: HÄTÄHYPPY!

- Instructor: YLÖS – KÄYTETÄÄN VARAVARJOA (UP – USE RESERVE CANOPY)!  Get up, look and grab the reserve handle.

- Instructor: OVELLE (DOOR)!  Set up to exit body position at the door, hold the reserve handle with your hand.

- Instructor: MENE (GO)!  Exit and maintain good arch. Do not perform checks or exit count at the door.

- Pull your reserve handle by both hands right after exit.

Emergency exit with main canopy

During levels 1-7 you jump emergency exit with main canopy with one instructor. Instructor is ready to assist with deployment. Starting from level 8, you’ll jump emergency exit alone when main canopy is used. In both cases deployment is performed right after exit.

Commands and actions when using main canopy

- Instructor commands: HÄTÄHYPPY (EMERGENCY EXIT)!  Repeat: HÄTÄHYPPY!

- Instructor: YLÖS – KÄYTETÄÄN PÄÄVARJOA (UP – USE MAIN CANOPY)!  Get up.

- Instructor: OVELLE (DOOR)!  Take exit body position at the door.

- Instructor: MENE (GO)!  Exit and maintain good arch. Exit checks or count is not performed.

- Deploy main canopy right after exit – Arch, reach, pull.

If instructor commands SEIS, PAKKOLASKU (STOP, EMERGENCY LANDING) stop, sit down, and follow actions on 5.1.3.

# 5.1.3 Emergency Landing (Pakkolasku)

Instructor is the chief of students ← Listen to his/her instructions!

Instructor commands PAKKOLASKU (EMERGENCY LANDING)!  Repeat: PAKKOLASKU!

- Sit down to a seat or floor your back facing towards the direction of travel and lean as tight as possible with others or plane structure.

- Buckle your seatbelt if possible.

- Link your arms with others.

Evacuation on the ground:

- When movement has stopped, exit the plane in order and as quickly as possible while avoiding the propeller.

- Assist injured people if possible.